13 Jul Plantaže  A.D.

- Place  and date : Podgorica, 10.10.2017

- PUBLIC INVITATION FOR SUBMISSION OF THE SAMPLES OF CORKSTOPPERS

I   Data on the Procurer

| Procurer: “13 Jul Plantaže” A.D. Podgorica | Contact person: Nenad Dulović |
| --- | --- |
| Address: Put Radomira Ivanovića br. 2 | Postal No: 81000 |
| Main office: Podgorica | Fiscal Ident.No (Comp.Reg.No): 02016281 |
| Tel: 00382 20 444 113 | Fax: 00382 20 658 025 |
| E-mail address: nenad.dulovic@plantaze.com | Internet page (web): www.plantaze.com |

To be submitted 100 pieces of the samples of corkstoppers for each item, by the following technical specification:

| Agglomerated corkstopper |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ord.No. | Description of the subject of procurement, ie of the part of the subject of the procurement | Important characteristics of the subject of the procurement concerning quality, performances and/or measures | Unit of measure | Quantity |  |
| 1 1 | Micro agglomerated corkstopper Corkstopper agglomerated with 2 discs 44 x 23,5 | Height: 44 +-0.5mm Diameter: 24 +- 0.3mm Elasticity (return of the corkstopper into the initial form after corking)≥97% Humidity: 4-8% Manufacture of the corkstopper: corkstopper is made of the sterilised microgranulate of granulometry 0,5- 1mm, of very low density,high flexibility and organoleptically neutral Density: 285+-30 kg/m3 Dust contents: <1 mg/corkstopper TCA presence:<0,5 ng/l per corkstopper Peroxide presence: < 0,2 mg/cork Stopper Keeping wine: up to 4 years in the normal conditions Logo of the company: Print Height: 44 ±1,0mm Width: 23,5+-0,4mm Weight of one corkstopper:4,8-5,7 g Specific weight: 250-330 kg/m3 Humidity: 4-8% Manufacture of corkstopper: the body of the corkstopper is made of the purified microgranulate, of neutral odour Discs, made of the natural cork, type kval-fleur (thickness of the discs min.4 mm) Peroxide presence:< 0,1 mg/corkstopper TCA presence: ≤ 2 ng/l Ovality of the corkstopper:≤ 0,3 mm Logo of the company: print/toasting Keeping wine: 4 years | pc | 100 |  |
| 2 | Corkstopper (natural cork 49 x 24) | Height: 49 +-0.5mm Diameter: 24 +- 0.4mm Weight of a corkstopper:2,8-5 g Specific weight:130-220 kg/m3 Humidity: 4-8% Manufacture of the corkstopper: made of the natural cork,type kval-fleur Peroxide presence: no residues TCA presence:<1ng/l Logo of the company: Print/toasting Keeping wine: 10 years | pc | 100 |  |
| 3 | Corkstopper (natural cork 44x24) | Height: 44 +-0.5mm Diameter: 24 +- 0.4mm Weight of a corkstopper:2,6- 4,5 g Specific weight:130-220 kg/m3 Humidity: 4-8% Manufacture of the corkstopper: made of the natural cork,type kval-fleur Peroxide presence: no residues TCA presence:<1ng/l Logo of the company: Print/toasting Keeping wine: 10 years | pc | 100 |  |

| Ord.No. | Description of the subject of procurement, ie of the part of the subject of the procurement | Important characteristics of the subject of the procurement concerning quality, performances and/or measures | Unit of measure | Quantity |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Corkstopper for sparkling wine | Height: 48 +-0.5mm Diameter: 30,7 +- 3mm Specific weight:230-290 kg/m3 Humidity: 4-9% Manufacture of the corkstopper: body of the corkstopper is made of the purified microgranulate. Discs are made of the natural cork (thickness of the discs-1: ≥ 4,5 mm; 2: ≥ 5,5 mm) Dust contents: < 2 mg/corkstopper Peroxide presence: < 0,1 mg/cork stopper TCA presence:<2 ng/l Logo of the company: Print/toasting | pc | 100 |  |
| 2 | Corkstopper with a plastic head | Type of the neck of the bottle:OB17 Total height: 28,5 +-0.2mm Height of the plastic part of the corkstopper: 8+-0,3 mm Diameter of the plastic part of the corkstopper: 27 +- 0,2 mm Diameter of the cork part of the corkstopper: 18+-0,2 mm Manufacture: Upper part is made of the hard plastics and lower part is made of the natural cork with rounded edges.The corkstopper is inert to high alcohol contents (cc 50 vol %)concerning spirits | pc | 100 |  |
| Senders of the samples are obliged to send together with the samples the following documents: - Declaration of the standard ,holding: ISO 9001, 22000 or HACCP - Reference list of the buyers for the concerned corkstoppers with whom they have co-operated in previous two years, signed and sealed by the Buyer - Recommendation of the manner of keeping, storage and use of the concerned corkstoppers - Producer’s specification |  |  |  |  |  |

The samples should be sent on working days from  07.00 to 14.00 hrs, closing on the 13th of November 2017 by 10.00 hrs.

Samples may be delivered by post or by direct handing at the Purchaser’s archive office,address Put Radomira Ivanovića No. 2, Podgorica.

The sender bears the costs  of samples and post fee.

All additional questions and information should be sent to e-mail: nenad.dulovic@plantaze.com and  tanja.djokovic@plantaze.com

NOTE:

The evaluation of the correctness of the quality of the submitted samples has the elimination character, meaning that only the bidders, the samples of which pass successfully the quality control, will be able to take part in the temders for the procurement of the concerned corkstoppers in the following period.

## DATA  ON  SENDER OF THE  SAMPLES

| Title and seat of the bidder |  |
| --- | --- |
| Fiscal Identification No |  |
| VAT |  |
| Address |  |
| Telephone |  |
| Fax |  |
| E-mail |  |
| Name and surname of the contact person |  |

Or National identification No of the country where the Bidder’s main office is located